The Circuit Layout

Some distance displacement for spring in order to get the maximum Force.

Experiment Set Up

February 21st, 2015

Today, I have tried to verify experimentally if properties of solenoids F0411A and G0403A correspond to their properties from Data Sheet.

In order to find these properties, I attached weights to the solenoids’ shafts and tested if they could hold the weight. The weights were attached vertically down from the solenoids’ shafts.

The shat itself weights = 8 grams

F0411A – Pontiac Coil (PL012DC) → Continuous

This solenoid has a continuous pull. According to the data sheet, it should have a sealed force of 41 oz., which is 1162 grams.

Experimentally Tested:

1158 g (weight) + 8 g (shaft)= 1166 g → did hold

1174 g (weight) + 8 g (shaft)= 1182 g → did not hold

As a result, experimentally results are very close to those from data sheet specifications.

G0403A – Pontiac Coil → Pulse

According to the data sheet, this solenoid should have a latched force of 37 oz., which is 1049 grams.

Experimentally Tested:

1628 g (weight) +8 g (shaft) = 1636 g → held for 20 seconds and it could more

1702 g (weight) +8 g (shaft) = 1710 g → held for 20 seconds and it could more

1769 g (weight) +8 g (shaft) = 1777 g → held for 18 seconds and it fell down

As a result, the tested results are higher than those from data sheet. Therefore, we might use this solenoid for our upcoming experiment.